1.	Overview

With CPU control and PWM technology, EBT series charger has the advantages like high efficiency, good stability, low-carbon and energy-saving. Three-section charging mode ---- Controlling the charging current and voltage automatically and precisely, both ensure that battery is charged fully, and not over charged. Your battery life will be protected.

2.	Unpacking Inspection

Open the package case, check the items shown on Table 1 carefully to check any missing or damaged part.

Table 1. Unpacking Inspection

| Item | Description | Qty |
| --- | --- | --- |
| 1 | Charger | 1 |
| 2 | Operating manual | 1 |
| 3 | AC wire | 1 |
| 4 | Output fuse | 2 |

3.	Charger Model & Parameter

3.1	Model series

◎120W   ◎180W   ◎240W   ◎360W

◎600W   ◎900W   ◎_____

3.2	Rated input voltage

◎110V          ◎220V         ◎110/220V

3.3	Constant voltage

◎12.6V   ◎14.6V   ◎18.2V   ◎29.4V   ◎42V

◎44.1V   ◎54.6V   ◎58.4V   ◎58.8V   ◎71.4V

◎73.0V   ◎73.5V   ◎84.0V   ◎87.6V   ◎_____

3.4	Floating charge voltage (For lead-acid battery)

◎27.6V   ◎41.4V     ◎55.2V   ◎________

3.5	Constant current

◎0.1～10.0A  ◎10.1～20.0A  ◎20.1～30.0A  ◎________

3.6	Light turn green

◎5-10%CC		   ◎20%CC	       ◎________

4.	Operating Instruction

Fig 1. Connection diagram AC input Charger Battery

4.1	Before charging, please connect the DC terminal to battery 	firstly, and then connect the AC terminal.

4.2	After charging complete, unplug the AC terminal firstly, and 	then unplug the DC terminal.

4.3	No charging or after charging complete, be sure to unplug the 	DC terminal and the AC terminal.

4.4	LED: the charging status indicator, shows red during the 	charger is charging, shows green means the battery is charged completely.

5.	Matters Need Attention

5.1	Don’t cover the charger or intake during charging.

5.2	The charger should be placed where children could not touch.

5.3	Please charging in indoor area where good ventilation and heat dissipation is. Prohibited charging where is humid, high temperature or a flammable, explosive gas surrounding.

5.4	To avoid vibration damage, please don’t carry the charger on the driving vehicle long time.

5.5	It’s danger of high voltage in charger case please do not disassemble it unauthorized.

5.6	The losses caused by not operating according to the instruction manual operation, manufacturer does not bear any responsibility.

6.	Maintenance

WARNING: Before conducting the following operations, make sure the charger is disconnected with the battery and power connection.

  - Routine maintenance

○	Clear dust on vents and fan regularly, to ensure the charger 	has good heat dissipation.

○	Periodically wipe the case with a cloth with some alcohol.

○	Do not use or store the charger in a place of humidity or    high temperature.

  - Replace the fuse

If the charger can not charge, please confirm the output fuse 	is blown out. Install only replacement fuse with the identical 	type and specification as follows and make sure the fuse is 	fixed firmly in the fuse holder.

This Operating Manual covers information on safety and cautions. Please read it carefully before charging

OPERATING

MANUAL